CCNA 2 Lab 1.5.1

Instructions: Download Lab 1.5.1 PacketTracer activity from Moodle.

Configure the network devices with the IP settings shown in Table 1.

Table 1:

Connect the Network devices according to the following network topology shown in Figure 1

Figure 1: Network topology for 1.5.1

| Task | Specification |
| --- | --- |
| Router name | R1 and R2 |
| R1 interfaces | - set the description - set IP address and Mask from table 1 |
| R2 interfaces | - set the description - set IP address and Mask from table 1 |
| PC1 and PC2 interfaces | set appropriate IP address and Mask from table 1 |
| R1 Static route | Configure static route that allow communication between the 192.168.1.0/24 and 192.168.3.0/24 Use R2 S0/0/0 as next hop IP, Use distance-metric 10 |
| R2 Default Static route | Configure static default route that allow communication between the 192.168.3.0/24 and any network Use R2 s0/0/0 as exit interface |

Router Commands

Configure a static route

Router(config)# ip route network-address subnet-mask {ip-address | exit-interface } distance-metric