CCNA 1 Chapter 5 activity: VLSM to increase security and performance

In this activity you are required to;

- Create an efficient VLSM design given the requirements.

- Assign appropriate addresses to interfaces and document the addresses.

- Cable a network according to the Topology Diagram.

You will create a small network that requires connecting network devices and configuring host computers for basic network connectivity.

Figure 1: Network Scenario

Task 1: Design the Logical LAN Topology.

Given an IP address and mask of 10.10.1.0 / 24 (address / mask), design a VLSM IP addressing scheme that satisfies the following requirements:

| Subnet | Number of Hosts |
| --- | --- |
| Subnet A | 8 |
| Subnet B | 56 |

Table 1: Requirements

The 0th subnet is used. All work must be shown.

Subnet B

| Specification | Student Input |
| --- | --- |
| Network Address | 10.10.1.0 |
| Subnet Mask (Decimal) | 255.255.255.192 |
| CIDR Notation | /26 |
| First IP Host address | 10.10.1.1 |
| Last IP Host address | 10.10.1.62 |
| Number of usable hosts in subnet | 64-2=62 |
| Broadcast address | 10.10.1.63 |

Table 2: Subnet B

Subnet A

| Specification | Student Input |
| --- | --- |
| Network Address | 10.10.1.64 |
| Subnet Mask (Decimal) | 255.255.255.240 |
| CIDR Notation | /28 |
| First IP Host address | 10.10.1.65 |
| Last IP Host address | 10.10.1.78 |
| Number of usable hosts in subnet | 16-2=14 |
| Broadcast address | 10.10.1.79 |

Table 3: Subnet A

Host computers will use the FIRST IP address in the subnet. The network router will use the LAST network host address.

Write down the IP address information for each device:

| Device | IP address | Mask | Gateway |
| --- | --- | --- | --- |
| PC 0 | 10.10.1.65 | 255.255.255.240 | 10.10.1.78 |
| PC 1 | 10.10.1.66 | 255.255.255.240 | 10.10.1.78 |
| Router0-Fa0/0 | 10.10.1.78 | 255.255.255.240 | No Gateway |
| PC 2 | 10.10.1.1 | 255.255.255.192 | 10.10.1.62 |
| PC 3 | 10.10.1.2 | 255.255.255.192 | 10.10.1.62 |
| Router0-Fa0/1 | 10.10.1.62 | 255.255.255.192 | No Gateway |

Table 4: Interface IP address information

Task 2: Configure the Logical Topology.

Open the file called Chapter 5 Topology.pka on the network drive.

Step 1: Configure host computers with correct IP configuration

Step 2: Configure Router0.

# Configuration tasks for Router0 include the following:

| Task | Specification |
| --- | --- |
| Router name | Router0 (Already set) |
| Console Password | Cisco (Already set) |
| Router0 interface Fa0/0 | - set IP address and Mask |
| Router0 interface Fa0/1 | - set IP address and Mask |

Table 5: Router configuration details

Hint: Use the commands sheet to help configure the router

Task 3: Verify network connectivity.

Network connectivity can be verified with the ping command.

Can PC 3 be pinged from PC 1? Yes, they are on the same router.